+-------------------------------------------------------------+

|                                                             |

|                                                       tm    |

|     ������  �������  �       ������  �������  �������       |

|     �       �     �  �       �       �     �  �     ��      |

|     �       �     �  �       �       �     �  �     ��      |

|     �       �������  �       ������  �     �  �     ��      |

|     �       �     �  �            �  �     �  �     ��      |

|     ������  �     �  ������  ������  �������  �������       |

|                                                             |

|                                                             |

|  COMPUTER-AIDED LOUDSPEAKER SYSTEM OPTIMIZATION AND DESIGN  |

|                                                             |

|               VERSION 3.10  AND  VERSION 1.40               |

|                                                             |

|                      3 February 1997                        |

|                                                             |

|       Demo Disk Instructions and Information Package        |

|                                                             |

|  (C) Copyright 1988-1997 by Audiosoft, All Rights Reserved  |

|                                                             |

+-------------------------------------------------------------+

AUDIOSOFT

128 ORIEL ROAD

WEST HEIDELBERG, 3081

MELBOURNE

AUSTRALIA

Fax: +61 3 9497 4441

Email: audiosoft@netwide.com.au

CONTENTS

~~~~~~~~

AVAILABILITY

PARTIAL LIST OF CALSOD USERS

EQUIPMENT REQUIREMENTS FOR RUNNING CALSOD

PRODUCT DESCRIPTION

Introduction

Powerful optimization capabilities

High performance analysis facilities

Modelling of non-coincident drivers

Use standard Thiele-Small parameters

Minimum-phase driver models

Computer-aided curve fitting

Improve your loudspeaker designs

Quality control applications

Usefulness in a teaching environment

ADDITIONAL FEATURES IN CALSOD 2.50 (ADVANCED EDITION)

THE CALSOD USER'S MANUAL

TYPICAL ANALYSIS PROCEDURE

Use manufacturers response data

Curve-fitting of data

Optimization of network components

GENERAL PROGRAM SPECIFICATIONS

USING MODULES TO CREATE A DATA FILE

THE CALSOD MENUS

RUNNING THE DEMO PROGRAM

THE CALSOD TEXT EDITOR

CALSOD EDITOR QUICK REFERENCE GUIDE

DEMONSTRATION CALSOD DATA FILE

Model of a two-way system

Crossover network circuit topology

Listing of sample data file

AVAILABILITY

~~~~~~~~~~~~

The CALSOD program is currently available from the following vendors.

VENDOR                        CALSOD EDITION

------                        --------------

Audiosoft                     All editions.

128 Oriel Road

West Heidelberg 3081

Melbourne

Australia

Fax: +61 3 9497 4441

Email: audiosoft@netwide.com.au

Old Colony Sound Lab          CALSOD 1.40 and CALSOD 3.10 with on-disk

PO Box 243                    user's manual.

Peterborough

NH 03458-0243

USA

Tel: +1 603-924-6371 or

+1 603-924-6526

Fax: +1 603-924-9467

Sonic Design HB               All editions.

PL 2448

820 41 Farila

Sweden

Tel:   +46 651 21355

Fax:   +46 651 23005

Email: sd@ljusdal.se

Harmonic Design GmbH          CALSOD 3.10 with printed user's manual.

Eberdinger Str. 37            A German-language edition of the manual

71665 Vaihingen/Enz           is available.

Germany

Tel: +49 7042-7086

Fax: +49 7042-78411

KEY Audio Development         CALSOD 1.40 and CALSOD 3.10 with on-disk

Karl-Marin-Str. 4B            user's manual.

D-21629 Nue Wulmstorf

Germany

Tel: +49 177-214-2352

Fax: +49 40-700-3691

Euterpe Audio S.A.R.L.        All editions.

4 Rue Thiers

94130 Nogent-sur-Marne

France

Tel: +33 1-4876-3474

Fax: +33 1-4877-0898

Falcon Acoustics Ltd          CALSOD 1.40 and CALSOD 3.10 with on-disk

Tabor House, Norwich Road     user's manual.

Mulbarton, Norwich

Norfolk NR14 8JT

United Kingdom

Tel: +44 (0508) 578272

ME Technologies               All editions.

P.O. Box 50

Dyers Crossing NSW 2429

Australia

Tel: +61 065-502254

Fax: +61 065-502341

Outline snc                   CALSOD 1.40 and CALSOD 3.10 with on-disk

Via Leonardo da Vinci 56      user's manual.

25020 Flero (BS)

Italy

Tel: +39 30-358-1341

Fax: +39 30-358-0431

Marton Music                 CALSOD  1.40 and CALSOD 3.10 with on-disk

5 Masterson Avenue           user's manual.

Read, Burnley

Lancs. BB12 7PL

England

Tel: +44 1282-773198

Fax: +44 1282-773198

PARTIAL LIST OF CALSOD USERS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Some of the many companies that are currently using CALSOD include the

following:

Barlow Associates        (Australia)

Eclat Products           (Australia)

Nova Sound               (Australia)

Regency College of TAFE  (Australia)

M.P. & C. PVBA           (Belgium)

Bell Northern Research   (Canada)

Novatel Communications   (Canada)

Electrolab               (Denmark)

F3/LYD                   (Denmark)

B & W Loudspeakers       (England)

Canon                    (England)

Carlsbro Electronics     (England)

NAD Electronics          (England)

Cator                    (Finland)

Gradient OY              (Finland)

Salora OY                (Finland)

Cabasse Electroacoustics (France)

Nevrocom                 (France)

Grundig GmbH             (Germany)

Nokia Graetz             (Germany)

Proraum GmbH             (Germany)

Solton Music GmbH        (Germany)

Willi Studer GmbH        (Germany)

Elina SA                 (Greece)

Chario Loudspeakers      (Italy)

Oyster SAS               (Italy)

Rank O'Connors           (Malaysia)

AJ Van den Hul BV        (Netherlands)

BNS Vandenberghe         (Netherlands)

Duran Audio              (Netherlands)

Omnitronics BV           (Netherlands)

Willich Gresel           (Netherlands)

G & L                    (Norway)

Philips Radio & Comm.    (Sweden)

PSDI Norden AB           (Sweden)

Marantz Company          (USA)

Magnepan Inc             (USA)

Precision Acoustics      (USA)

Snell Acoustics          (USA)

Sterling Sound           (USA)

EQUIPMENT REQUIREMENTS FOR RUNNING CALSOD

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In order to run CALSOD you will require the following:

- IBM PC (or PC/XT or PC/AT) computer or "true" compatible using an

8088/8086/80286/80386/80486/Pentium/Pentium Pro processor. The

software is supplied on 3.5" floppy disks.

- CALSOD will make use of the 8087/80287/80387 maths coprocessor if

one is installed. This will speed up the computational speed

considerably.

- PC-DOS or MS-DOS version 3.00 or higher. The CALSOD program also

runs in a DOS session under Windows 95 and Windows NT.

- A hard disk with at least 1.6 MB of free space is required for

installation of the CALSOD 1.40 program. Up to 3.5 MB of disk

space is required by CALSOD 3.10 (on-disk user's manual edition)

depending on the installation options.

- 512K of free RAM.

NOTE: If your free memory is less than this, especially if you

have loaded a number of memory resident utilities, you may need

to reconfigure your system.

- Colour Graphics Adapter (CGA), Enhanced Graphics Adapter (EGA),

or the Hercules Graphics Card (HGC) for IBM PC computers, or

Video Graphics Array (VGA) for IBM PS/2 computers.

Note that CALSOD will plot out response curves in color when an

EGA/VGA graphics card and monitor combination are used.

- Graphics monitor capable of working with the appropriate graphics

card.

- Printer support for HP LaserJet laser printers, HP DeskJet inkjet

printers, and 9-pin and 24-pin dot matrix printers from IBM,

Epson, NEC and Toshiba is provided in both CALSOD 1.40 and

CALSOD 3.10 versions.

NOTE: The demo-disk version supports only 9-pin Epson/IBM

compatible dot matrix printers.

PRODUCT DESCRIPTION

~~~~~~~~~~~~~~~~~~~

Introduction

~~~~~~~~~~~~

CALSOD (which stands for Computer-Aided Loudspeaker System

Optimization and Design) is a program for use in the computer-aided

design and optimization of loudspeaker systems that use passive

crossover networks.

The CALSOD program is now available in three versions. The advanced

edition is CALSOD 3.10, and this includes a 360-page User's Manual

housed in an A4 size 3-ring binder, where all the options in the

modules and submodules are described in detail. The standard edition

is CALSOD 1.40, which includes an on-disk User's Manual running to

about 240 formatted pages, now also provided as Windows Write 3.1 and

Word for Windows 2.0 files as well as formatted ASCII text files.

There is also a special edition of CALSOD 3.10 that comes with the

complete User's Manual on-disk. A set of formatted ASCII text files

are provided, as well as a file in Windows Write 3.1 format, and

another in Word for Windows 2.0 format. A complete set of printed

figures is supplied. Because of the significant savings in air mail

postage costs, as well as printing costs, we can offer this version of

CALSOD 3.10 at a significantly reduced cost. However, because the

User's Manual is a lengthy document, it is recommended that you have

access to a laser printer or a fast inkjet printer for best results.

All these User's Manuals describe the design of a complete two-way

system in a tutorial-like fashion, thus ensuring that you can quickly

develop the skills required to carry out computer-aided analyses of

loudspeaker systems. All the sample data files described in the User's

Manual are provided on disk, and they can be used as the starting

point for your own designs.

Note that the features common to both editions of CALSOD will be

described shortly, and those capabilities that are incorporated only

in the advanced edition will also be described.

We would like to highlight that those users who now purchase

CALSOD 1.40 will be able to upgrade to CALSOD 3.10 for simply the

difference in price. This includes the full printed version of the

User's Manual. Note that the data file format used by CALSOD 3.10 is a

superset of that used in CALSOD 1.40, so any data files that you

create will not need to be modified should you ever decide to upgrade.

Programs incorporating the capabilities provided within CALSOD have

previously only been available to large manufacturers who have access

to minicomputer or mainframe computing facilities. These manufacturers

can create optimized loudspeaker systems with much less design effort

than the smaller manufacturers, many of whom cannot afford the expense

of investing in a large computer and software development costs.

Now CALSOD brings a collection of powerful design capabilities within

reach of all engineers, technicians, and hobbyists engaged in the

design of loudspeaker systems. The program runs on the IBM PC/XT/AT

family of personal computers, as well as the IBM PS/2 series

computers, and it will also run on many close compatibles. The

software is menu driven, thus making it easy to learn and use.

In particular, we would like to emphasize that CALSOD's ability to

predict and optimize the summed acoustic response of multiway

loudspeaker systems is unique in a PC-based CAD program. In fact, many

of CALSOD's capabilities are so new that an article describing some of

the modeling techniques that CALSOD uses will has been published in

the Journal of the Audio Engineering Society ("Simulation and

Optimization of Multiway Loudspeaker Systems Using a Personal

Computer," J. Audio Eng. Soc., Vol. 36, No. 9, 1988 September,

pp. 651-663).

After reviewing the CALSOD package, the following comments were made

by Vance Dickason, author of "The Loudspeaker Design Cookbook" and

editor of "Voice Coil", in his user report:

+============================================================+

| "... the program performs a lot of slick tricks. One of    |

| the more spectacular of these is allowing the designer to  |

| specify the location of driver acoustic centers using an   |

| XYZ coordinate system."                                    |

|------------------------------------------------------------|

| "The 133-page User Manual is well written and adequately   |

| describes the various program  functions and contains an   |

| excellent tutorial example, which demonstrates the use of  |

| the program."  (Note: the latest manual has been expanded) |

|------------------------------------------------------------|

| "Obviously, CALSOD is an extremely flexible tool that will |

| allow you to move into many uncharted areas, and perform   |

| the kind of system modelling which has never before been   |

| possible on a desktop computer."                           |

|------------------------------------------------------------|

|  From Voice Coil, PO Box 176, Peterborough, NH 03458, USA. |

|          (c) Copyright 1988. All rights reserved.          |

+============================================================+

The January 1989 issue of the magazine Elektor Electronics (UK

edition) contained a review of CALSOD. Some of the comments made by

the reviewer are given below:

+============================================================+

|    "Other software packages for loudspeaker design and     |

|    optimizing, offering a price/performance tradeoff at    |

|    least equal to that of CALSOD, are, to the best of      |

|    our knowledge, not available."                          |

|------------------------------------------------------------|

|     "The redesign feature of the program was tested on     |

|     existing loudspeaker systems. Remarkably, CALSOD's     |

|     computed response was found to correspond exactly      |

|     with the measured response."                           |

|------------------------------------------------------------|

|   "CALSOD is capable of optimizing the complete system,    |

|   working effectively towards the realization of the       |

|   target response."                                        |

|------------------------------------------------------------|

|   "CALSOD is a well-designed and remarkably practical      |

|   program that will prove invaluable to the designer ... " |

|------------------------------------------------------------|

|     "... this tool greatly simplifies formerly often       |

|     tedious and time-consuming work."                      |

|------------------------------------------------------------|

|     "The optimizing procedure can provide really good      |

|     results, and the options for analysing a complete      |

|     system are unique."                                    |

|------------------------------------------------------------|

|       From Elektor Electronics, 1 Harlequin Avenue,        |

|               Brentford TW8 9EW, England.                  |

|         (c) Copyright 1989. All rights reserved.           |

+============================================================+

The November 1990 issue of the magazine Stereophile contained a

speaker design article written by Dick Olsher. Some of the comments

about CALSOD that were made in that article are given below:

+============================================================+

|    "I cannot imagine tackling another speaker project      |

|    without such a computer-aided design tool as CALSOD."   |

|------------------------------------------------------------|

|   "CALSOD includes an outstanding user's manual. It is     |

|   extensive and worth its weight in gold. There are        |

|   plenty of design examples, proceeding from the simple    |

|   toward the complex. Installation, the structure of the   |

|   program, the input file format, and all of the input     |

|   modules are very well explained."                        |

|------------------------------------------------------------|

|   "CALSOD enabled me to narrow down the field from a       |

|   bewildering array of crossover possibilities without     |

|   having to spend time and money chasing down dead ends.   |

|   It is a fantastic design tool that affords the speaker   |

|   builder flexibility and design sophistication previously |

|   available only to industry giants."                      |

|------------------------------------------------------------|

|   "Listening tests and measurements with the Neutrik       |

|   system confirmed that CALSOD's predictions were quite    |

|   close to the mark."                                      |

|------------------------------------------------------------|

|      "This is a mature and polished program, a product     |

|      typical of what one would expect from a large         |

|      software-development house."                          |

|------------------------------------------------------------|

|     "The real potency of CALSOD lies in its ability to     |

|     optimize the summed acoustic response of the entire    |

|     driver array."                                         |

|------------------------------------------------------------|

|             From Stereophile, 208 Delgado,                 |

|               Santa Fe, NM 87501, U.S.A.                   |

|         (c) Copyright 1990. All rights reserved.           |

+============================================================+

FEATURES IN CALSOD 1.40 (STANDARD EDITION)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Powerful optimization capabilities

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

By using CALSOD's optimization capability, the speaker designer can

create crossover networks in a fraction of the time it would normally

take to go through the design and testing cycle. Also, a much more in-

depth analysis can be carried out at the design stage, resulting in

both an improved design and significant savings in testing time.

CALSOD allows the designer to optimize circuit components for the

following types of problems:

- Single driver/filter combinations can be optimized to a yield a

desired acoustic target function.

Pre-defined Butterworth, Linkwitz-Riley, and constant voltage

response functions are available. User-defined target transfer

functions can also be specified. High-pass and low-pass filter

functions can be cascaded to form target band-pass response

characteristics.

- Multiple driver/filter combinations are optimized on the basis of

a specified target summed acoustic response. This is generally a

flat frequency-independent response specified over a given

frequency range.

The formulation of the optimization problem that CALSOD uses has the

advantage that deviations in the summed magnitude response caused by

phase or magnitude response errors will be reduced during the

optimization. This will generally result in a more optimal solution

than can be attained using methods that can only optimize individual

drivers. Optimization of the summed response is also beneficial when a

particular rolloff rate and crossover frequency are required. In such

cases one part of a low- and high-pass filter pair is optimized

separately, and then, with the components of the second part allowed

to vary, the summed response is optimized while the components of the

first part are held fixed.

The circuit optimizer will take your network specification and vary

the components that have been labelled as variable. Some of these may

prove to be redundant, which is evident when the optimizer returns

component values that are very large or very small. This occurs

because the optimizer takes into account the natural rolloff of the

driver in the vicinity of the crossover frequency.

High performance analysis facilities

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Using CALSOD's advanced analysis facilities, you can routinely inspect

both the magnitude and phase of additional performance parameters such

as:

- impedance of each loudspeaker,

- input impedance of the crossover network and loudspeaker

combination,

- voltage response of each filter when it is loaded by its driver,

- filtered and unfiltered sound pressure response of each

loudspeaker,

- desired target acoustic response function, and

- summed sound  pressure response  of a multiway loudspeaker system

comprised of a number of individual drivers.

All of these response functions are a simple menu selection away. By

using CALSOD's integrated graphics display facilities, you can

routinely display plots of all the above characteristics, both before

and after carrying out an optimization, thus making it easy to

evaluate and validate the performance of any proposed design. Since

plots of the magnitude and phase response can also be presented using

a linear frequency scale, this permits the prediction of the phase

delays present in the summed response of multiway loudspeaker systems,

thus allowing the linear-phase behaviour of the design to be assessed.

Modelling of non-coincident drivers

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

An important feature of CALSOD is that it can take into account the

effects of acoustic phase due to geometric offsets in the positions of

the acoustic centres of loudspeaker drivers in a multiway system. The

relevant data is input via the SPK submodule, which is described later

in the Information Package. By using the TARGET module the designer

specifies the coordinates of the design point where the outputs of the

individual drivers are summed, and CALSOD automatically includes all

the inter-driver time delays. This feature allows the designer to

simulate loudspeaker systems that use both coincident and

noncoincident drivers, the latter being the most commonly available.

Without this capability, the loudspeaker designer cannot determine how

well his design will perform in real life. Most other programs simply

assume that the acoustic centers of the drivers are coincident, which

can lead to significant errors in the summed response at the design

point. CALSOD allows you to inspect this critical performance

criterion for every design you produce, giving you the information you

require to evaluate the success of a design.

Note that the CALSOD User's Manual specifically includes an example

where the summed response of a two-way system with non-coincident

drivers is simulated and optimized. Also, the data file that is

described later corresponds to the optimization of a two-way system

with non-coincident drivers.

Use standard Thiele-Small parameters

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

CALSOD uses the industry standard Thiele-Small parameters supplied by

the manufacturer of the loudspeaker drivers. With these values you can

specify low-frequency alignments for vented-box, closed-box, passive-

radiator, band-pass, and filter-assisted loudspeaker systems. Both the

sound pressure and impedance response curves can be simulated for all

these systems.

The use of Thiele-Small parameters is adequate in most circumstances.

However, some drivers do not always follow the theoretical ideal. To

cope with these, CALSOD also allows you to incorporate minimum-phase

equalization into the model, thus getting an improved match between

experimental data and the simulation.

Minimum-phase driver models

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The real power of CALSOD lies in your ability to create a complete

model of a multiway loudspeaker system and to evaluate and modify its

response characteristics to meet a given set of design goals. The

capabilities of CALSOD are analogous to modern finite element packages

used in structural analysis where a complete structure is assembled

from a number of discrete elements. CALSOD allows you to manually

create your own library of individual loudspeaker drivers. Drivers can

be selected from this library and incorporated into the current design

project. In this way new designs can be quickly created to meet the

requirements of an expanded product range. Also, old designs can be

easily modified to include new drivers that will improve the

performance of the loudspeaker system.

These unique features result from the ability of CALSOD to develop

accurate minimum-phase models of the complex impedance and sound

pressure response of moving-coil loudspeaker drivers. These models are

a general representation of a particular driver, and once defined they

are used as building blocks to create a complete model of a multiway

loudspeaker system. If desired, the low-frequency alignments can be

changed from one design to the next.

By the application of minimum-phase equalization functions, even

drivers with peaks and dips in their response characteristics can be

accurately simulated. This ensures that both the magnitude and phase

response will be accurately represented by the model.

Computer-aided curve fitting

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The techniques used to create these models are based on the

application of interactive computer-aided curve-fitting techniques.

These methods are easy to learn and use, yet provide a powerful and

flexible model of a loudspeaker driver which can then be interfaced

with the design and analysis capabilities of CALSOD. In order to apply

these methods the user need only have available the magnitude response

of the driver sound pressure and impedance functions, data which is

provided by nearly all manufacturers of loudspeaker drivers.

Once the curve-fitting process has been completed for the driver, the

simulation will model both the magnitude and the phase of the sound

pressure and impedance transfer functions.

Improve your loudspeaker designs

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Using the computer-aided loudspeaker design capabilities of CALSOD has

a number of other significant advantages. By developing transfer

function models of the behaviour of individual loudspeaker drivers,

the speaker designer can check the results of the optimization without

having to measure experimentally the response of the loudspeaker

system. It is no longer necessary to have access to costly

instrumentation and a suitable measurement environment throughout the

design process, although it is still desirable that the final design

should be verified experimentally.

By using computer-aided analysis, the designer can also evaluate many

more proposals to see which one best achieves the specified

performance objectives. Once the models of the driver's have been

established, it is easy to try out alternative physical layouts of the

drivers in the loudspeaker system, which may be particularly important

when three or more drivers are used.

Quality control applications

~~~~~~~~~~~~~~~~~~~~~~~~~~~~

As a result of variations in the manufacturing processes involved in

the production of loudspeaker drivers, the sound pressure response of

a particular model of driver is likely to vary slightly from batch to

batch. When such variations occur, the existing transfer function

model can be easily modified to match the characteristics of the

current batch of drivers, and then the crossover networks of any

loudspeaker systems using that driver can be re-optimized using the

original design as the initial trial solution. The revised component

values can then be incorporated into the assembly process, and it is

possible that producing revised filter components may prove more cost

effective than ensuring strict uniformity of loudspeaker driver

performance from one production batch to another.

CALSOD also generates an input impedance curve for the complete

loudspeaker system composed of multiple drivers and their crossover

network. This can be compared with the measured input impedance of the

manufactured product, thus ensuring correct operation of this critical

component.

Rapid design of air-cored inductors

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

A utility program is provided to enable the crossover designer to

quickly and easily determine the number of turns required to achieve a

desired inductance when using an air-cored coil. Both square and

circular bobbins are catered for. The dc resistance of the coil is

also calculated, assuming that copper wire is being used.

Usefulness in a teaching environment

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Furthermore, because of its broad range of standard analysis

capabilities, CALSOD can be a productive learning tool at all levels

of instruction in loudspeaker system design. It also provides a good

introduction to the use and effectiveness of computerized optimization

methods. This makes CALSOD particularly useful in a University or

Technical College teaching environment because students are easily

able to investigate various designs, thus significantly enhancing the

learning process. The low-cost standard edition of CALSOD is

particularly attractive.

ADDITIONAL FEATURES IN CALSOD 3.10 (ADVANCED EDITION)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The advanced edition, CALSOD 3.10, has a number of additional features

which enhance its simulation and optimization capabilities. These are

as follows:

- The summed  sound pressure  response can  now be calculated for a

combination of  on-axis and  off-axis  observation  points,  with

overlaid color  plots  making  it  a  simple  matter  to  compare

response curves.

- The optimizer has been extended and now has the ability to take

into account the off-axis summed response at multiple observation

points. This means that your designs will have a more uniform on-

axis and off-axis response, since the components of the crossover

will be automatically varied to achieve this (if desired by the

designer, of course).

- Impedance equalization/correction networks, also known as

conjugate load matching networks, can be optimized directly. The

easy to use procedure is described in two sample problems in the

User's Manual.

- Active crossover networks are now also supported, and they are

modelled using ideal operational amplifiers. Active filters can

be optimized just the same as networks that were made up of

passive filter components.

- A new feature incorporated into CALSOD's simulation capabilities

is the ability to model each driver as a circular piston. This

allows the designer to include the effects of the driver's off-

axis radiation characteristics, and this capability will

considerably enhance the accuracy of the simulations. This virtue

is particularly important when dealing with non-coincident

drivers, and it complements the ability of CALSOD's optimizer to

take into account on-axis and off-axis sound pressure responses

during the optimization process.

- It is possibly to directly import data obtained from audio

measurement equipment such as DRA Laboratories' MLSSA, Ariel's

SysID, Audio Precision's System One, and LinearX Inc's LMS,

Liberty Instrument's IMP and Audiosuite, Audiomatica's CLIO,

Bunn's AIRR, Microacoustics' PC Audiolab, and Kemsonic's AMS-PC.

This feature saves a lot of time and enhances the accuracy of the

simulations. Multiple off-axis response conditions can be

utilized by the optimizer.

- It is possible to estimate the Thiele-Small and fundamental

parameters of a driver using two sets of impedance measurements

taken under mass loading or enclosure loading conditions. The

driver's voice-coil inductance is also estimated using the

frequency-dependent parameters.

- A single impedance measurement of the driver when mounted in a

vented-box of known volume can also be used to estimate the

driver's Thiele-Small and fundamental parameters. This technique

provides a means for checking the alignment of a given enclosure

to see whether it is performing correctly. The estimated Thiele-

Small parameters are used to set up a model of the sound pressure

response of vented system, and the resulting response plot can be

viewed on-screen for easy verification.

THE CALSOD USER'S MANUAL

~~~~~~~~~~~~~~~~~~~~~~~~

Each of the CALSOD User's Manuals explains how to apply the advanced

simulation and optimization capabilities of CALSOD to the design of

multiway loudspeaker systems. The new user is led through the design

of a two-way loudspeaker system, and once this material is mastered it

can be applied to the design and analysis of other systems. Some of

the areas covered in the manual are:

- full descriptions of all the modules and submodules that are used

in creating a data file to simulate and optimize a multiway

loudspeaker system

- how to create models of the impedance response of loudspeaker

drivers, including frequency dependent resistive and reactive

terms for modelling voice-coil inductance

- how to create models of the sound pressure response of

loudspeaker drivers, including the peaks and dips that are

present in the response of many drivers

- design procedure for analysis and optimization of multiway

systems with coincident and non-coincident driver acoustic

centers

- design of systems with vented-box, closed-box, passive-radiator,

band-pass, or filter-assisted low-frequency alignments

- initial design of passive filter networks prior to optimization

including the use of standard filter formulas for estimating

initial component values

- initial design of active filter networks prior to optimization

including the use of standard filter formulas for estimating

initial component values (CALSOD 3.10 only)

- design and optimization of impedance equalization networks

(CALSOD 3.10 only)

- optimizing the target sound pressure using multiple on-axis and

off-axis summed responses (CALSOD 3.10 only)

- importing of measurement data from acoustical test systems and

its use in design and optimization, including the use of

measurements taken at multiple observation points (CALSOD 3.10

only)

- design of the port for vented-box systems

- modelling of the baffle diffraction step for a driver on a baffle

- modelling of the floor reflections

- extensive bibliography covering articles on loudspeaker enclosure

and crossover network design

TYPICAL ANALYSIS PROCEDURE

~~~~~~~~~~~~~~~~~~~~~~~~~~

The analysis procedure that is recommended to the user of CALSOD

consists of two distinct parts. The first part involves creating the

transfer function models of the sound pressure and impedance responses

of the individual drivers that are to be used in the loudspeaker

system. In the second part of the analysis, the filter topology is

chosen and the optimization is carried out if desired.

Use manufacturers response data

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The raw data used to approximate the sound pressure and impedance

response of a loudspeaker driver is generally obtained from the

appropriate frequency response plots provided by the manufacturer. If

the impedance response is not available then it will need to be

measured or estimated in some way. The Thiele-Small parameters can be

used for this purpose. As a result of the minimum-phase behaviour of

most moving-coil loudspeaker drivers, only the magnitude of the sound

pressure and impedance responses are required. The minimum-phase

property ensures that the phase response is correctly represented if

the magnitude response is accurately modelled. If phase data are

available then they should be utilized in order to provide an

additional check on the accuracy of the approximation that is being

developed.

Curve-fitting of data

~~~~~~~~~~~~~~~~~~~~~

Once the raw data has been collected, analytical transfer function

approximations to the sound pressure and impedance responses of the

chosen drivers are created using a computer-aided curve-fitting

process. During this process, the designer compares his analytical

model with the raw data using CALSOD's graphics capability. The models

are successively modified until the desired accuracy has been

achieved. At this stage of the analysis you may wish to introduce your

own low-frequency alignments into the models using the modules related

to vented-box, closed-box, passive-radiator and band-pass systems.

These are based on the Thiele-Small parameters as well as additional

data such as the voice-coil resistance and inductance.

Optimization of network components

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Once satisfactory sound pressure and impedance response transfer

function approximations have been developed, a filter network topology

is chosen for the combination of drivers. The order of the filters is

generally in the range from first-order to fourth-order, and the

choice will be based on the designer's requirements concerning power

handling, polar response, power response and phase response. The

initial component values of the chosen filter network are often based

on handbook values obtained from filter tables, and some design

formulas are presented in the CALSOD User's Manual.

After the trial crossover network has been specified, an optimization

can be performed in one of two different ways:

a. Optimization of the summed acoustic response to yield the desired

target response (available in both editions of CALSOD). If

desired, this can take into account the on-axis and off-axis

response characteristics at multiple observation points

(CALSOD 3.10 only).

b. Optimization of the individual low-pass and high-pass (or

bandpass) filters to achieve the desired target responses. The

target responses chosen at this stage are generally known to

yield the required response when the outputs of the individual

drivers are combined.

Complete transfer function models of the sound pressure and impedance

behaviour of individual drivers can often be defined in a matter of

minutes, especially once the designer has gained some experience. If

the driver has a large number of peaks and dips in its sound pressure

response, the curve-fitting process may take longer. However, once the

model has been defined it can be used in many different analyses

without having to be re-defined.

Once the initial filter component values have been assigned,

optimization of the crossover network can proceed. If only one driver

is being optimized, a solution is generally obtained within a few

minutes. A filter with 6 or so components that can vary during the

optimization can generally be optimized in less than 10 minutes (all

times are for an IBM PC/XT running at 4.77 MHz). The exact time taken

will depend on the number frequency points that are used.

Optimization of the summed response generally takes longer, mainly

because the circuit contains a greater number of components, and also

because the interactions are potentially more complex. Again, the time

taken will depend on (1) the number of components that are allowed to

vary, and (2) the characteristics of the system being optimized.

Typical times will be in the range 10 to 20 minutes. For example, a

crossover network for a three-way system with 15 components (10 were

variable) took about 13 minutes to optimize.

NOTE: If an 8xx87 maths coprocessor is installed, then all of the

calculations and optimizations will be performed very much

faster. Hence, for serious design work, a maths coprocessor is

strongly recommended. Also, modern 80486 or Pentium computers

will be able to carry out the calculations and optimizations in

seconds, rather than minutes.

GENERAL PROGRAM SPECIFICATIONS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Some general features of the CALSOD program are given below:

- up to 60 crossover components can be defined, not including the

SPK submodules that are used in the CIRCUIT module. This permits

networks with large numbers of components to be analyzed, as

found in multiway loudspeaker systems.

- the maximum number of variable components that can be optimized

is also 60.

- up to 100 frequency points can be used when calculating the

system transfer functions and carrying out the optimization. This

allows adequate resolution when creating plots.

- Multiway systems with up to 8 drivers can be simulated, and these

can consist of up to 8 different drivers as defined in the DRIVER

modules. For example, a 7-driver 4-way system comprised of

2 woofers, 2 low-midrange drivers, 2 high-midrange drivers, and

1 tweeter, can be simulated using the standard CALSOD

configuration.

USING MODULES TO CREATE A DATA FILE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Data is input into CALSOD in a modular form, with each module

beginning with a name. Within each module there are a number of

different submodules which are used to define the characteristics of

that particular module. For example, the program uses the CIRCUIT

module to define the passive filter network that will be used as the

crossover. The CIRCUIT module can contain the submodules RES, IND,

CAP, and SPK, which are used to define resistors, inductors,

capacitors, and individual loudspeakers that are connected within the

filter network.

Note that the tutorial examples in the User's Manuals carefully guide

the new user through the design of a two-way system, including the

simulation of a system with non-coincident drivers. This means that

you will quickly become familiar with and proficient in the creation

of quite complex system models.

The following are short descriptions of the capabilities of each of

the modules and submodules that can be used to set up a data file that

models a loudspeaker system. Any features that apply only to

CALSOD 3.10 are noted as such in the text.

Also presented is the format that the data takes when it is used in a

data file. However, space does not permit us to describe all of the

parameters that are used with each submodule, although the

nomenclature that we have adopted will already be familiar to many

loudspeaker designers. All the relevant terms are described in the

CALSOD User's Manual, together with appropriate examples of their use.

APD

~~~

This submodule is used to modify the calculation of the propagation

delays from each loudspeaker driver to each observation point. This

option allows you to compensate for any air path delays that are

measured by your acoustical analysis equipment (such as FFT

analyzers), thus making it easier to compare CALSOD's simulations with

the measured results.

APD  Value  METRES/SECONDS

BDS

~~~

This submodule is used to simulate the effects of baffle diffraction

on the sound pressure response of a driver mounted in a loudspeaker

enclosure.

BDS  Width/Frequency  METRES/HZ  BOOST/LOSS

BES

~~~

Used to introduce the response characteristic of a Bessel filter.

High-pass and low-pass filter responses can be defined.

BES  Fo  Order  HIGHPASS/LOWPASS

BSL

~~~

Used to introduce the response characteristic of a set of Bessel

filter alignments that can be summed to provide a reasonably flat

magnitude response. High-pass and low-pass filter responses can be

defined.

BSL  Fo  Order  HIGHPASS/LOWPASS

BUT

~~~

Used to introduce the response characteristic of a Butterworth filter.

High-pass, low-pass and all-pass filter responses can be defined. (The

ALLPASS option is available only in CALSOD 3.10).

BUT  Fo  Order  HIGHPASS/LOWPASS/ALLPASS

CAP

~~~

Used to introduce a capacitor into the crossover network circuit

topology.

CAP  Value  Nlow  Nhigh  FIXED/VARIABLE

CIRCUIT

~~~~~~~

This module is used to signal the start of data defining the crossover

network and the connection of any loudspeaker drivers. An example of a

complete CIRCUIT module is provided in the sample data file located at

the end of this document.

CVB    (CALSOD 3.10 only)

~~~

Used to specify the box volume and the effective diaphragm area of the

loudspeaker driver for use in determining the driver's Thiele-Small

parameters from a single impedance measurement of the driver mounted

in a vented-box enclosure.

CVB  Vbox  Sd

CVB  Vbox  Diam  Eratio

DRIVER

~~~~~~

This module is used to define the data for the sound pressure and

impedance response approximations to the behaviour of a loudspeaker

driver. The model of the driver can be incorporated as an element in

the crossover network and, if desired, the same driver may be used

more than once.

EPD    (CALSOD 3.10 only)

~~~

This module is used to define the effective piston diameter of the

driver, which will be used when calculating off-axis sound pressure

response of the driver

EPD  Diameter

EXPERIMENTAL IMPEDANCE

~~~~~~~~~~~~~~~~~~~~~~

This module is used to read in experimental impedance data so that the

data points can be plotted. This permits the easy comparison of the

raw data with the impedance response transfer function approximation

that is being developed. CALSOD 3.10 allows the user to directly

import measured data from System One, MLSSA, LMS or SysID data files.

EXPERIMENTAL SPL

~~~~~~~~~~~~~~~~

This module is used to read in experimental sound pressure level data

so that the data points can be plotted. This permits the easy

comparison of the raw data with the sound pressure response transfer

function approximation that is being developed. CALSOD 3.10 allows the

user to directly import measured data from System One, MLSSA, LMS or

SysID data files.

FIL

~~~

Used to introduce the response characteristics of complementary high-

pass and low-pass filter responses that have been defined for many

common filter functions (Butterworth, Linkwitz-Riley and constant-

voltage designs). If the desired filter function cannot be called up

using this module then you must define your own filter response using

the TFN module.

FIL  Fo  FilterNo  HIGHPASS/LOWPASS

FILE    (CALSOD 3.10 only)

~~~~

This submodule is used to read in sound pressure or impedance response

data contained in a file. With this submodule, it is possible to make

direct use of data obtained from different audio measurement systems.

The data in the file can be arbitrarily scaled and adjusted for any

time delays that may be present in the measurements. The various file

formats that are supported include those used by Audio Precision's

System One, Ariel's SYSid, DRA Laboratories' MLSSA, Liberty

Instrument's IMP, Audiomatica's CLIO, and LinearX Inc's LMS.

FILE Format Name Dfit Dextra Dtype Factor Ftype

HLR

~~~

Used to create a sound pressure response function corresponding to a

loudspeaker driver in a closed-box enclosure or in free-air. The

response corresponds to a second-order (12 dB/octave cutoff) high-pass

filter function. It is possible to include the effects of the voice-

coil inductance.

HLR  Fs  Qvc  Qms  Qes  Alpha

IAC

~~~

This submodule is used to introduce an air-cored inductor into the

crossover network circuit topology. It is assumed that copper wire is

used for the coil. The user can input either the wire diameter in

metres, or simply specify the SWG of the wire that is being used. When

an IAC submodule is used to specify the inductor, CALSOD automatically

computes the parasitic dc resistance of the coil based on the user-

supplied dimension information in the parameter DSWG. Depending on the

magnitude of the value of DSWG, the appropriate formula is selected

from the two that are given in the section defining the IND submodule.

IAC  Value  Nlow  Nhigh  DSWG  Type

IBP

~~~

Used to create an impedance curve corresponding to a loudspeaker

driver in a band-pass enclosure. If desired, the effects of the voice-

coil inductance can be simulated. Additional options are also provided

to enable frequency-dependent resistive and reactive parameters to be

used when modelling the voice-coil inductance, making the model much

more accurate.

IBP  Fs  Rvc  Lvc  Qms  Qes  Alpha 1 Alpha2  h  QL Qp Qb  Delta

ICB

~~~

Used to create an impedance curve corresponding to a loudspeaker

driver in a closed-box enclosure. The resulting impedance curve has a

single peak which is a characteristic of a driver in a closed-box. If

desired, the effects of the voice-coil inductance can be simulated.

Additional options are also provided to enable frequency-dependent

resistive and reactive parameters to be used when modelling the voice-

coil inductance, making the model much more accurate.

ICB  Fs  Rvc  Lvc  Qms  Qes  Alpha

ICQ    (CALSOD 3.10 only)

~~~

Similar to the ICB submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses.

ICR

~~~

Used to create an impedance curve corresponding to a constant

resistance.

ICR  Value

IDP    (CALSOD 3.10 only)

~~~

Similar to the IBP submodule, except that additional parameters are

available for modelling the impedance curve for a double ported

bandpass enclosure. Leakage, absorption, and port losses can be

separately specified for each enclosure.

IMP

~~~

Used to create an impedance curve with a peak at frequency Fo. The

effects of the voice-coil inductance can also be included. The IMP

submodule can be used to model individual loudspeaker drivers that are

in free-air or are mounted in a closed-box. Additional options are

also provided to enable frequency-dependent resistive and reactive

parameters to be used when modelling the voice-coil inductance, making

the model much more accurate.

IMP  Fo  Rvc  Zmax  Lvc  Qt

IND

~~~

Used to introduce an inductor into the crossover network circuit

topology. It is possible to specify a value for the series resistance

of the inductor in order to model the losses present in practical

coils.

IND  Value  Nlow  Nhigh  DCR  FIXED/VARIABLE

IOA    (CALSOD 3.10 only)

~~~

Used to include an operational amplifier into the crossover network.

IOA  N1  N2  N3

IPQ    (CALSOD 3.10 only)

~~~

Similar to the IPR submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses, as

well as passive radiator losses.

IPR

~~~

Used to create an impedance curve corresponding to a passive-radiator

loudspeaker system. The resulting impedance curve has the twin peaks

which are a characteristic of this design. Additional options are also

provided to enable frequency-dependent resistive and reactive

parameters to be used when modelling the voice-coil inductance, making

the model much more accurate.

IPR  Fs  Rvc  Lvc  Qms  Qes Alpha  H  QL  Qmp  Delta

IVB

~~~

Used to create an impedance curve corresponding to a loudspeaker

driver in a vented-box enclosure. The resulting impedance curve has

the twin peaks which are a characteristic of a driver in a vented-box.

IVB  Fs  Rvc  Lvc  Qms  Qes  Alpha  H  QL

IVQ    (CALSOD 3.10 only)

~~~

Similar to the IVB submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses, as

well as port losses.

LKR

~~~

Used to introduce the response characteristic of a Linkwitz-Riley

filter. High-pass, low-pass and all-pass filter responses can be

defined.

LKR  Fo  Order  HIGHPASS/LOWPASS/ALLPASS

MPE

~~~

Used to introduce minimum-phase equalization to the sound pressure and

impedance transfer function approximations created to model a

particular loudspeaker driver. By using the MPE submodule you can

accurately represent the peaks and dips that may be present in the

driver's performance characteristics.

MPE  Fo  Qeq  Deq  DB/LINEAR

MRX    (CALSOD 3.10 only)

~~~

Used to introduce a minimum-phase frequency-dependent impedance

component into the crossover network topology. This is handy for

accurately modelling a driver's voice-coil impedance at high

frequencies when creating equivalent circuit models of a loudspeaker

driver.

MRX  Kmp  Xmp  Nlow  Nhigh  VARIABLE/FIXED

OCC

~~~

This submodule is used to introduce an open circuit into the crossover

network circuit topology. The use of an open circuit is convenient if

we wish to remove a component from the network without affecting the

node numbering.

OCC  Nlow  Nhigh

PEC    (CALSOD 3.10 only)

~~~

This submodule is used to specify the conditions associated with a set

of two impedance measurements that have been taken in order to

estimate the Thiele-Small parameters of a driver. Either mass loading

or enclosure loading conditions can be used.

PEC  M1  M2  Vbox1  Vbox2  Sd

RAB    (CALSOD 3.10 only)

~~~

Used to define off-axis observation points for calculation of the

summed response. (R,Alpha,Beta) are specified using a spherical

coordinate system. The submodule is made ACTIVE if the summed response

at this observation point is to be taken into account by the

optimizer, and the WtVal parameter specifies the relative weighting of

each of the active submodules.

RAB  R  Alpha  Beta  WtVal  ACTIVE/INACTIVE

RGE    (CALSOD 3.10 only)

~~~

Used to create an equalization function that can be used to model the

low-frequency boost in sound pressure response that occurs when a

loudspeaker is placed close to room boundaries. Two different room

gain functions are presently supported.

RGE  FuncNum

SBP

~~~

Used to create a sound pressure response function corresponding to a

loudspeaker driver in a band-pass enclosure.

SBP  Fs  Qts  Alpha1  Alpha2  h  QL  Qp  Qb  Delta

SCQ    (CALSOD 3.10 only)

~~~

Similar to the SCB submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses.

SCB

~~~

Used to create a sound pressure response function corresponding to a

loudspeaker driver in a closed-box enclosure. This is a second-order

(12 dB/octave cutoff) high-pass filter function.

SCB  Fo  Qts  Alpha

SCC

~~~

This submodule is used to introduce a short circuit into the crossover

network circuit topology. The use of a short circuit is convenient if

we wish to remove a component from the network without affecting the

node numbering.

SCC  Nlow  Nhigh

SDP    (CALSOD 3.10 only)

~~~

Similar to the SBP submodule, except that additional parameters are

available for modelling the sound pressure response for a double

ported bandpass enclosure. Leakage, absorption, and port losses can be

separately specified for each enclosure.

SEN

~~~

Used to define a sensitivity value which can be selected to match the

sensitivity of the loudspeaker driver. It can also be used to adjust

the overall level of the target function for the optimization

procedure. It is also possible to input a set of Thiele-Small

parameters or fundamental driver parameters to establish the

sensitivity of the driver.

SEN  Sens  DB/LINEAR

SEN  Re    Fs   Qes   Vas   TSP

SEN  Re    Bl   Sd    Mms   FDP

SEN  Re    Nopc %

SOF

~~~

Used to create a sound pressure response function corresponding to a

second-order filter response characteristic (12 dB/octave cutoff).

Both high-pass and low-pass filter characteristics can be modelled.

Different filter response shapes can be obtained by varying the Q of

the filter. When used in conjunction with the closed-box (SCB) and

vented-box (SVB) submodules, it is possible to model filter-assisted

low-frequency alignments such as the closed-box B4 and vented-box B6.

SOF  Fo  Q  HIGHPASS/LOWPASS

SPK

~~~

Used to introduce a loudspeaker driver into the circuit topology. By

defining different values of (X,Y,Z) coordinates it is possible to

model the position of the acoustic centre of the loudspeaker driver on

the enclosure baffle. If the coordinates are varied from one

simulation to the next, then the effects on the summed acoustic sound

pressure of placing the driver in different locations can be

determined. In CALSOD 3.10 it is also possible to specify the skew

angles of the principal axis of radiation of the driver when mounted

on the baffle (denoted by Alpha and Beta below).

SPK  DriNo  Nlow  Nhigh  X Y Z  POSITIVE/NEGATIVE

* SPK  DriNo  Nlow  Nhigh  X Y Z  Alpha  Beta  POSITIVE/NEGATIVE

* indicates this is a CALSOD 3.10 feature only

SPQ    (CALSOD 3.10 only)

~~~

Similar to the SPR submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses, as

well as passive radiator losses.

SPR

~~~

Used to create a sound pressure response function corresponding to a

passive-radiator loudspeaker system.

SPR  Fo  Qts  Alpha  H  QL  Qmp  Delta

SVB

~~~

Used to create a sound pressure response function corresponding to a

loudspeaker driver in a vented-box enclosure. This is a fourth-order

(24 dB/octave cutoff) high-pass filter function.

SVB  Fo  Qts  Alpha  H  QL

SVQ    (CALSOD 3.10 only)

~~~

Similar to the SVB submodule, except that additional parameters are

available for modelling enclosure absorption and leakage losses, as

well as port losses.

TARGET

~~~~~~

This module is used to signal the start of data defining the target

sound pressure response that will be used during the optimization

procedure. Although all the submodules that can be used in the TARGET

module define sound pressure functions that have both magnitude and

phase, only the magnitude of the target response is utilized during

the optimization procedure.

TARGET IMPEDANCE    (CALSOD 3.10 only)

~~~~~~~~~~~~~~~~

This module is used to signal the start of data defining the target

impedance response that will be used during the optimization

procedure. Although all the submodules that can be used in the TARGET

module define impedance functions that have both magnitude and phase,

only the magnitude of the target response is utilized during the

optimization procedure.

TFN

~~~

Used to introduce an arbitrary user-defined transfer function into the

model. Therefore it is possible for you to incorporate a

characteristic that is not supplied as standard with CALSOD.

TFN   Fo    NoNum  NoDen

N1 N2 N3 .. NNoNum

D1 D2 D3 .. DNoDen

TITLE

~~~~~

This module is used to introduce up to four lines of text which will

be used to label the bottom of plots when you generate screen dumps on

your printer.

TITLE  NumOfLines  ENABLE/DISABLE/BOTTOM

Line 1

Line 2

Line NumOfLines

VIN

~~~

Used to specify the voltage being applied to the input terminals of

the crossover network circuit. If the VIN submodule is omitted, then

it is assumed that the input voltage is set to the standard value of

2.828 volts, which corresponds to an input power of 1 watt into a load

of 8 ohms.

VIN  Value

XYZ

~~~

Used to input the coordinates of the point in space at which the

target response is defined.

XYZ  X  Y  Z

THE CALSOD MENUS

~~~~~~~~~~~~~~~~

CALSOD is a menu driven program that presents you with a table of

options during each phase of the analysis. In order to carry out some

operation, such as reading in a new data file or starting the

optimization procedure, you are required to make a selection by

pressing the key whose character denotes the desired process. Some

operations result in a submenu being displayed, from which you are

once again required to make a selection.

When you start CALSOD the following main menu will appear on the

screen. By choosing from the available options it is possible to carry

out the design, analysis, and optimization of a loudspeaker system.

The main menu that appears in CALSOD 1.40 and CALSOD 3.10 is shown

below:

+=====================================================================+

|                    C  A  L  S  O  D                     | 1:49:03 pm|

|Computer-Aided Loudspeaker System Optimization and Design|26 JAN 1997|

|(C) Copyright by Audiosoft, 1988-97, All Rights Reserved |296 Kb free|

|=====================================================================|

|                                                                     |

|            A   Invoke editor                                        |

|            B   Plot system responses                                |

|            C   List response tables                                 |

|            D   Directory of files                                   |

|            E   Change frequency range and optimizer parameters      |

|            F   Change plotting parameters                           |

|            G   List component values                                |

|            H   Begin optimization procedure                         |

|            I   Read in new data file                                |

|            J   Change target SPL coordinates                        |

|            K   Change target SPL sensitivity                        |

|            L   Change active directory                              |

|            M   Recalculate system transfer functions                |

|            N   Exit program                                         |

|                                                                     |

|        Select command letter :                                      |

|                                                                     |

+=====================================================================+

RUNNING THE DEMO PROGRAM

~~~~~~~~~~~~~~~~~~~~~~~~

With your computer running, and the DOS prompt showing, you should

insert the demo disk in Drive A. In order to start the CALSOD

demonstration program you should enter the following commands at the

DOS prompt:

A:

CD \

CALSOD

Plotting response functions

~~~~~~~~~~~~~~~~~~~~~~~~~~~

Once the initial display appears, you will be prompted to press any

key to continue. After you have done so, a dummy data file will be

read in and the system transfer functions will automatically be

calculated. After the Main Menu appears you can select Option B to

plot out and view all the various functions. If you select Option B

you will be prompted to select from a table of choices all the plots

you wish to have displayed.

NOTE: It is not possible to simultaneously display plots of sound

pressure and impedance. If after choosing Option B from the main menu

you happen to select both a sound pressure function and an impedance

function, CALSOD will only display the impedance functions that were

selected; the sound pressure functions will be omitted.

Optimizing the crossover network

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When the demo program starts up, the number of subintervals is

automatically set to 50, and the lower and upper limits of the

frequency range are set to 20 Hz and 20 kHz, respectively.

For the optimization to operate correctly, only about 20 subintervals

in the frequency range from 1 kHz to 20 kHz need to be specified. It

is possible to have more subintervals but this only slows down the

optimization process.

Note that the complete CALSOD package gives guidelines and examples in

the User's Manual as to how to go about selecting an appropriate

frequency range and number of subintervals for the optimization.

In order to change the number of subintervals and the frequency range,

select Option E from the Main Menu. Select the appropriate sub-options

to change the number of subintervals to 20, and the frequency limits

to 1 kHz and 20 kHz.

After doing this you will be asked whether or not to recalculate the

system transfer functions, and since the default answer is Yes you

should simply press the Return key and the process will be initiated.

After it is completed the Main Menu will reappear, and you can select

Option B if you wish to take a look at plots of the various functions.

By selecting Option H you can begin the optimization. Once it becomes

evident that there is little or no improvement in the Current Error

(which is a parameter that is displayed during the optimization), you

should press the Esc key to stop the optimization. Note that

convergence to a solution occurs in about 10 iterations. By selecting

Option B from the main menu and then choosing the plot you wish to

have displayed, you will be able to view the filtered responses of the

tweeter and woofer after the filter network components have been

optimized to achieve the best fit to the target function (a flat

frequency-independent all-pass response).

Once the optimization has been completed, you can change the number of

subintervals to about 50, and the frequency range to be between 20 Hz

and 20 kHz, so that the plots will have a smoother appearance. If you

increase the number of subintervals after the optimization has been

carried out with only 20 subintervals, you will need to recalculate

all the system transfer functions.

THE CALSOD TEXT EDITOR

~~~~~~~~~~~~~~~~~~~~~~

The CALSOD text editor is supplied with the complete CALSOD package in

order to permit you to create and modify data files in a way that is

integrated with the analysis capabilities of CALSOD. It can be used to

create and change the data files which define the problem that is

being set up to be solved.

Some of the features of the text editor are:

- The command interface is similar to that of another popular word

processor. The various commands are presented in the section

"CALSOD Editor Quick Reference Guide".

- There is an Undo facility, invoked by pressing the Esc key while

you are in the edit mode, that allows you to restore the text to

its state before a deletion.

- It is possible to display the files that are contained in any

directory.

- The facility to rename, copy and delete disk files is provided.

- A complete pulldown menu system, invoked by pressing the

F10 function key, simplifies the operation of the editor.

- You can mark a block of text and then copy, move or delete it. It

is also possible to write the selected block to a disk file.

- The facility to find a pattern of characters, and replace it with

another pattern if desired, is available.

- It is possible to open a second window in order to copy text from

it to the main file that you are editing. This is useful for

creating new data files using elements that have been defined in

other data files.

It is not possible to invoke the editor in this demonstration version

of CALSOD. If you attempt to do so, then a message telling you of the

editor's unavailability will be displayed.

CALSOD EDITOR QUICK REFERENCE GUIDE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

^A      Go left one word          ^Q S  Jump to far left on line

^S      Go left one character     ^Q D  Jump to far right on line

^D      Go right one character    ^Q J  Jump to marker

^F      Go right one word         ^Q I  Toggle auto indent mode

^E      Go up one line            ^Q R  Go to top of file

^X      Go down one line          ^Q C  Go to end of file

^R      Go up one page            ^Q B  Go to beginning of block

^C      Go down one page          ^Q K  Go to end of block

^W      Scroll up one line        ^Q 1  Jump to marker 1

^Z      Scroll down one line      ^Q 2  Jump to marker 2

RETURN  New line                   ...  ....

^N      Insert blank line         ^Q 9  Jump to marker 9

^G, DEL Delete character          ^Q Y  Delete line right

BKSP    Delete left character     ^Q F  Find text pattern

^I      Tab                       ^Q A  Find and replace text

^T      Delete word

^Y      Delete line               ^K S  Save file and resume

^K D  Save and open file

^V      Toggle insert mode        ^K X  Exit to CALSOD

^L      Find next occurrence      ^K Q  Abandon and open file

^J      Beginning/end of line     ^K 1  Set marker 1

^P      Insert char by ASCII      ^K 2  Set marker 2

...  ....

^O O    Open new window           ^K 9  Set marker 9

^O L    Set left margin           ^K B  Begin block

^O R    Set right margin          ^K K  End block

^K H  Hide block

^O K    Change case               ^K C  Copy block

^O S    Set UNDO limit            ^K V  Move block

^O W    Toggle wordwrap mode      ^K Y  Delete block

^O G    Go to other window        ^K W  Write block

^O I    Go to column              ^K R  Read block

^O N    Go to line                ^K M  Set marker

^O Y    Destroy window            ^K T  Define tab width

ESC     Undo last change

F10     Activate pulldown menus

Cursor controls

~~~~~~~~~~~~~~~

Up      Go up one line

Dn      Go down one line

Lft     Go left one character     ^Lft   Go left one word

Rgt     Go right one character    ^Rgt   Go right one word

PgUp    Go up one page            ^PgUp  Go to top of file

PgDn    Go down one page          ^PgDn  Go to bottom of file

Home    Go to beginning of line   ^Home  Go to top window

End     Go to end of line         ^End   Go to bottom window

DEMONSTRATION CALSOD DATA FILE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

In this section we will describe the general characteristics of a

CALSOD data file so that you can get some idea of its features.

The drivers in most high performance loudspeaker systems are spaced in

such a way that the distances from their acoustic centers to the

listening point are different. Depending on the filter characteristics

and the degree of noncoincidence, the summed sound pressure response

at the design point can be significantly affected by the time delays

caused by the geometric offsets.

Because of the different times taken for sound waves to travel from

each loudspeaker to the listening or design point, sometimes these

geometric offsets are referred to as time offsets.

When considering the possible effects of geometric offsets, it is

useful to consider the amount of offset present in terms of fractions

of the wavelength corresponding to the crossover frequency. For

example, at 3 kHz a 25 mm offset corresponds to 0.22 of a wavelength,

and leads to a phase shift of 78 degrees, which is quite significant.

However, at 300 Hz this distance corresponds to only 0.02 of a

wavelength, and leads to an almost negligible phase shift of

8 degrees. Hence, although the distance is the same in both cases, the

parameter of significance is the offset as a function of wavelength.

CALSOD permits you to define the geometric locations of the acoustic

centers of the drivers on the baffle of the loudspeaker enclosure.

Consequently, by specifying the location of the design point it is

possible to predict the performance of the system with the effects of

the geometric offsets included. If desired, an optimization of the

summed response can also be carried out.

Model of a two-way system

~~~~~~~~~~~~~~~~~~~~~~~~~

As an example, we will describe a data file for a two-way system whose

drivers are non-coincident. The layout of the two drivers is shown in

the diagram below, and is typical of many woofer/tweeter combinations

that are mounted on a flat vertical baffle with their face-plates

coplanar.

Y-axis          All dimensions in metres

|             ~~~~~~~~~~~~~~~~~~~~~~~~

|

+    Tweeter  (X,Y,Z)=(0.0,+0.100,0.0)

|

|

|

------------�------------ X-axis

/|

/ |

/  | +  Woofer  (X,Y,Z)=(0.0,-0.100,-0.025)

/   |/

/    |

/

+ Design point  (X,Y,Z)=(0.0,0.0,2.000)

/

/

Z-axis

Both drivers are assumed to be facing in the positive Z-direction. The

design point at which the summed acoustic output of these two drivers

is evaluated corresponds to a point on the Z-axis that is 2.000 metres

from the origin of the coordinate system.

Crossover network circuit topology

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The circuit topology and node numbering convention for the crossover

network for the two-way system made up of a woofer and tweeter is

shown below:

+--+

1-+--|L1|------2-------+

|  +--+      |       |

|          +-+-+     |

|          |R1 |     |

|          +-+-+     |+         Inductor  L1 =  0.62E-03 H

|            |    +--+---+

|            3    |WOOFER|      Capacitor C1 = 11.97E-06 F

|            |    +--+---+

|          +-+-+     |-         Resistor  R1 = 14.45 ohms

|          |C1 |     |

|          +-+-+     |

|            |       |

|   0--------+-------+

|

|  +--+     +--+

+--|C2|--4--|R2|-----5

+--+  |  +--+     |+         Inductor  L2 =  0.28E-03 H

+-+-+     +---+---+

|L2 |     |TWEETER|      Capacitor C2 =  6.25E-06 F

+-+-+     +---+---+

|           |-         Resistor  R2 =  0.69 ohms

0----------+-----------+

Listing of sample data file

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The following is a listing of the data file that is read in by the

demo version of CALSOD. This data file is used as an initial design

for the optimization of the crossover network for the two-way

loudspeaker system. Note that in this example the acoustic centers of

the woofer and tweeter are not coincident. This data file cannot be

edited.

Note that the CALSOD User's Manual gives a complete description of the

modelling techniques used to create the models of the woofer and

tweeter, as well as describing how to model multiway loudspeaker

systems.

01> ! -----------------------------------------------------------

02> ! SAMPLE DATA FOR A TWO-WAY SYSTEM WITH NONCOINCIDENT DRIVERS

03> ! -----------------------------------------------------------

04> !

05> ! Optimization frequency range        : 1.0 kHz to 20.0 kHz

06> ! Optimization number of subintervals : 20

07> !

08> CIRCUIT

09> !

10> VIN  6.0  DB

11> !

12> ! Filter network for the woofer

13> !

14> IND  0.62E-3  1 2   0.00               VARIABLE

15> RES 14.45     2 3                      VARIABLE

16> CAP 11.97E-6  0 3                      VARIABLE

17> SPK   1       0 2   0.0 -0.100 -0.025  POSITIVE

18> !

19> ! Filter network for the tweeter.

20> !

21> CAP  6.25E-6  1 4                      VARIABLE

22> IND  0.28E-3  0 4   0.00               VARIABLE

23> RES  0.69     4 5                      VARIABLE

24> SPK   2       0 5   0.0  0.100  0.000  POSITIVE

25> !

26> TARGET

27> XYZ     0.0  0.0  2.000

28> SEN    92.0  DB

29> !

30> DRIVER 1

31> Woofer

32> SOUND PRESSURE

33> SEN    92.0    DB

34> SVB    33.7    0.27    4.0     1.5   7.0

35> MPE   580.0    2.65   -1.10    DB

36> MPE  2000.0    1.82    3.00    DB

37> BUT  4350.0    4       LOWPASS

38> IMPEDANCE

39> IVB    33.7   6.10   6.63E-4  6.34   0.28  4.0  1.5  7.0

40> MPE   420.0   3.95   1.175    LINEAR

41> MPE  1000.0   0.59   1.486    LINEAR

42> MPE  2000.0   1.61   1.050    LINEAR

43> MPE  3.5E+4   0.71   0.689    LINEAR

44> MPE  3.0E+5   0.70   0.219    LINEAR

45> !

46> DRIVER 2

47> Tweeter

48> SOUND PRESSURE

49> SEN      90.0    DB

50> HLR     535.0    0.0     3.14     0.43    0.0

51> MPE    1200.0    1.36    1.00     DB

52> MPE    3500.0    7.40    2.00     DB

53> MPE    6000.0    2.56   -1.25     DB

54> MPE    9000.0    8.17    1.00     DB

55> MPE   12000.0    1.13    1.25     DB

56> MPE   15000.0    5.65    1.30     DB

57> MPE   17000.0    6.20    0.25     DB

58> MPE   18200.0    7.86    0.50     DB

59> BUT   25000.0      4     LOWPASS

60> IMPEDANCE

61> IMP     535.0   6.10    34.80    0.0     0.55

62> MPE    2000.0   4.29     1.31    LINEAR

63> MPE    3800.0   2.00     1.05    LINEAR

64> MPE   16000.0   0.48     1.33    LINEAR

62> MPE   16000.0  12.00     1.14    LINEAR

63> MPE  500000.0   0.49     6.31    LINEAR

The following is a description of the major features of the data file

which is shown above.

Line 01-07 : These lines contain some comments that describe the data

contained in the file.

Line 08    : The CIRCUIT module header signifies the start of the data

that defines the crossover network.

Line 09    : Comment line used for text spacing purposes.

Line 10    : The VIN submodule is used to specify the value of the

input voltage applied to the input terminals. The 0 dB

reference voltage is 2.83 V.

Line 11-13 : Comments.

Line 14-16 : The IND, RES and CAP submodules specify the values of

inductor L1, resistor R1 and capacitor C1 shown in the

circuit diagram above. These three components make up the

filter network acting on the woofer, and all the

components are allowed to vary during the optimization

process.

Line 17    : This SPK submodule specifies the connection of the woofer

shown in the circuit diagram above. The woofer is

connected with positive polarity, and the geometric

location of the woofer is specified as

(X,Y,Z)=(0.0,-0.100,-0.025) metres.

Line 18-20 : Comments.

Line 21-23 : The CAP, IND and RES submodules specify the values of

capacitor C2, inductor L2 and resistor R2 shown in the

circuit diagram above. These three components make up the

filter network acting on the tweeter, and all the

components are allowed to vary during the optimization

process.

Line 24    : This SPK submodule specifies the connection of the

tweeter shown in the circuit diagram above. The tweeter

is connected with positive polarity, and the geometric

location of the tweeter is specified as (X,Y,Z)=(0.0,

0.100, 0.000) metres.

Line 25    : Comment line used for text spacing purposes.

Line 26    : The TARGET module header signifies the start of the data

that defines the target response.

Line 27    : This XYZ submodule specifies the (X,Y,Z) coordinates

where the summed acoustic sound pressure response of the

tweeter and woofer will be monitored. By setting

(X,Y,Z)=(0.0, 0.0, 2.000) the program will include the

effects of path length differences when calculating the

summed acoustic sound pressure response.

Line 28    : This SEN submodule defines a reference sensitivity for

the target function. The sensitivity is set to what is

thought to be an achievable value for the woofer/tweeter

combination. Since no other response functions are

specified, the target response is simply a flat

frequency-independent response characteristic.

Line 29    : Comment line used for text spacing purposes.

Line 30    : The DRIVER module header signifies the start of the

transfer function data used to define the sound pressure

and impedance response of the woofer. The driver code

number for the woofer is 1, and this is referred to in

the CIRCUIT module (see line 17).

Line 31    : This is the title or description of the woofer that will

be used to label various features of the output that are

relevant to this woofer. The title must be included

immediately after the DRIVER module header (no comment

lines are permitted). The title is used by CALSOD to

display a quick prompt you are when choosing the various

output options available.

Line 32    : The SOUND PRESSURE statement in the DRIVER module

signifies that the data which follows corresponds to the

sound pressure response of the woofer.

Line 33    : This SEN submodule defines the sensitivity of the woofer.

Line 34    : The SVB submodule defines the low-frequency response

characteristic of the woofer in a vented box.

Line 35-36 : These two lines are used to introduce minimum-phase

equalization into the sound pressure transfer function

(using the MPE submodule). Note the use of the "DB"

option to signify that the equalization is defined using

the decibel scale.

Line 37    : The BUT submodule introduces the high-frequency rolloff

that is present in the woofer sound pressure response.

The rolloff corresponds to a fourth-order Butterworth

low-pass response function.

Line 38    : The IMPEDANCE statement in the DRIVER module signifies

that the data which follows corresponds to the impedance

response of the woofer.

Line 39    : The IVB submodule sets up the impedance response of the

woofer when it is in a vented-box enclosure. Note that

the submodule includes the effects of voice-coil

inductance.

Line 40-44 : These five MPE submodules introduce minimum-phase

equalization of the impedance function defined in line

39.

Line 45    : Comment line used for text spacing purposes.

Line 46-63 : These lines define the tweeter sound pressure and

impedance response functions. The driver code number for

the tweeter is 2, and this is referred to in the CIRCUIT

module (see line 24). As the format of the data for the

tweeter simulation is very similar to that used for the

woofer, no additional descriptions will be given.

======================================================================

/------------------------\

| AUDIOSOFT  ORDER  FORM |

\------------------------/

AUDIOSOFT, 128 ORIEL ROAD, WEST HEIDELBERG 3081, MELBOURNE, AUSTRALIA

EMAIL: audiosoft@netwide.com.au

FAX: +61 3 9497 4441

+---------------------------------------+-------+---------+----------+

|                                       |       |  UNIT   |   SUB    |

|        PRODUCT DESCRIPTION            | UNITS |  PRICE  |  TOTAL   |

|---------------------------------------+-------+---------+----------|

| CALSOD 3.10 (Advanced edition)        |       |         |          |

| with 350-page printed User's Manual.  |       | $449.00 |          |

| Including air mail postage.           |       |         |          |

|---------------------------------------+-------+---------+----------|

| CALSOD 3.10 (Advanced budget edition) |       |         |          |

| with on-disk User's Manual (in ASCII  |       |         |          |

| text format, Windows Write 3.1 format,|       | $369.00 |          |

| and Word for Windows 2.0 format),     |       |         |          |

| with a set of printed figures.        |       |         |          |

| Including air mail postage.           |       |         |          |

|---------------------------------------+-------+---------+----------|

| CALSOD 1.40 (Standard edition)        |       |         |          |

| with on-disk User's Manual.           |       | $119.00 |          |

| Including air mail postage.           |       |         |          |

+---------------------------------------+-------+---------+----------|

TOTAL (Australian $) |          |

+----------+

SHIP TO: (Please use CAPITAL letters.)

~~~~~~~~

Name : _______________________________________________________

Company : _______________________________________________________

Address : _______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

_______________________________________________________

PAYMENT ENCLOSED:

~~~~~~~~~~~~~~~~~

[ ] VISA   [ ] MASTERCARD  [ ] Bank draft  [ ] International money

order

Card holder's name : _______________________________________________

Account number : _______________________________________________

Card expiry date : _______________________________________________

Signature : _______________________________________________

Date : _______________________________________________

PLEASE NOTE:

~~~~~~~~~~~~

CALSOD 1.40 and CALSOD 3.10 are available only on 1.44 MB 3.5" floppy

disks.

Payment must accompany your order.

VISA or MASTERCARD credit card numbers must consist of 16-digits.

Please make sure that you include the card holder's name, the full

account number and the expiry date with your credit card order, as

well as the authorized signature.

If paying by cheque, please ensure that the payment is in Australian

dollars, and have the cheque drawn on an Australian bank. Many of

our customers successfully used the ANZ Bank (Australian and New

Zealand Bank) and Commonwealth Bank. Other banks with local offices

in Melbourne or Sydney are also suitable.