The smallest handheld automatic one-chip AVR-based DC voltmeter

Project Number A3780

All programs written for IAR embedded Workbench for Atmel AVR

MAIN PROGRAM

// The smallest handheld automatic one-chip AVR-based DC voltmeter

#include <iom8.h>

#include <inavr.h>

#include "defines.h"

#include "cportlcd.h"

//FUSES: SET TO 0 SUT0, CKSEL1, CKSEL3 ONLY (EXT RC GENERATOR ~37kHz)

// f3 a3   b3 f2 a2 b2   f1 a1 b1 grad    com1

// e3 g3   c3 e2 g2 c2   e1 g1 c1 minusH  com2

// 00 d3   00 00 d2 DP    M d1 BT minusL  com3  	BT means battery symbol

// CONSTANTS TABLE FOR LCD DIGIT1

__flash TWord Character_set_digit1 [10][3]= //10digits*3words (com1 com2 com3)

{

{ 0x380, 0x280, 0x100 }, //0

{ 0x080, 0x080, 0x000 }, //1

{ 0x180, 0x300, 0x100 }, //2

{ 0x180, 0x180, 0x100 }, //3

{ 0x280, 0x180, 0x000 }, //4

{ 0x300, 0x180, 0x100 }, //5

{ 0x300, 0x380, 0x100 }, //6

{ 0x180, 0x080, 0x000 }, //7

{ 0x380, 0x380, 0x100 }, //8

{ 0x380, 0x180, 0x100 }, //9

};

// CONSTANTS TABLE FOR LCD DIGIT2

__flash TWord Character_set_digit2 [10][3]= //10digits*3words (com1 com2 com3)

{

{ 0x070, 0x050, 0x030 }, //0

{ 0x010, 0x010, 0x010 }, //1

{ 0x030, 0x060, 0x030 }, //2

{ 0x030, 0x030, 0x030 }, //3

{ 0x050, 0x030, 0x010 }, //4

{ 0x060, 0x030, 0x030 }, //5

{ 0x060, 0x070, 0x030 }, //6

{ 0x030, 0x010, 0x010 }, //7

{ 0x070, 0x070, 0x030 }, //8

{ 0x070, 0x030, 0x030 }, //9

};

// CONSTANTS TABLE FOR LCD DIGIT3

__flash TWord Character_set_digit3 [10][3]= //10digits*3words (com1 com2 com3)

{

{ 0x00e, 0x00a, 0x004 }, //0

{ 0x002, 0x002, 0x000 }, //1

{ 0x006, 0x00c, 0x004 }, //2

{ 0x006, 0x006, 0x004 }, //3

{ 0x00a, 0x006, 0x000 }, //4

{ 0x00c, 0x006, 0x004 }, //5

{ 0x00c, 0x00e, 0x004 }, //6

{ 0x006, 0x002, 0x000 }, //7

{ 0x00e, 0x00e, 0x004 }, //8

{ 0x00e, 0x006, 0x004 }, //9

};

TByte LCD_com_counter=0;

TWord StartMaskCom=0;

volatile TWord sleepCounter=0;

volatile TWord adcCounter;

volatile TWord previousADC;

bool beep;

TWord DataCom1=0;

TWord DataCom2=0;

TWord DataCom3=0;

TByte lcdBuffer[3];

void DeviceInit(void);

void HighBeep(void);

void LowBeep(void);

void LCD_UpdateBuffer(void)

{

DataCom1=Character_set_digit1[lcdBuffer[0]][0];

DataCom1|=Character_set_digit2[lcdBuffer[1]][0];

DataCom1|=Character_set_digit3[lcdBuffer[2]][0];

DataCom2=Character_set_digit1[lcdBuffer[0]][1];

DataCom2|=Character_set_digit2[lcdBuffer[1]][1];

DataCom2|=Character_set_digit3[lcdBuffer[2]][1];

DataCom3=Character_set_digit1[lcdBuffer[0]][2];

DataCom3|=Character_set_digit2[lcdBuffer[1]][2];

DataCom3|=Character_set_digit3[lcdBuffer[2]][2];

DataCom3|=StartMaskCom;

}

void DeviceInit(void)

{

__disable_interrupt();

//init ports

DDRB=0x03F;			//0 - INPUT, 1 - OUTPUT

PORTB=0;

DDRC=0x03B;

PORTC=0;

DDRD=0x0FF;

PORTD=0;

// init timer0

TCCR0=0x02;	//SET PRESCALER TO 8

TIMSK=0x01;	//ENABLE INT FROM TIMER0

//init timer1

TCCR1A=0xC2;	//MODE 14 (FAST PWM WITH ICR1)

//disable the internal comparator

ACSR=1<<ACD;

// ADC 2-nd channel; internal REF 2.56v

ADMUX=0xc2;

//XTAL/32+first dummy conversion

ADCSR=(1<<ADEN)|(1<<ADSC)|(1<<ADPS0);

__enable_interrupt();

}

void HighBeep(void)		//HIGH FREQUENCY BEEP BY PWM

{

TWord k;

TCCR1B=0x19;

ICR1=15;

for (k=0; k < 300; k++ );

TCCR1B=0;

}

void LowBeep(void)		//LOW FREQUENCY BEEP BY PWM

{

TWord k;

TCCR1B=0x19;

ICR1=30;

for (k=0; k < 300; k++ );

TCCR1B=0;

}

void ConvertResult(TWord v)

//for calibration: all values in raw ADC code

//then use the formula (((U+) - (U-))*10 / ADCmax - ADCmin) for calculating multiplier k

//then for calculating c in formula b=kx+c: c=(U+)*10 - k*ADCmax

{

float b;

TWord delta;

//CHECK FOR ACTIVE WORK WITH SLEEP COUNTER RESETTING (FOR AUTO POWER DOWN)

if ( v>previousADC )

{

delta=v-previousADC;

}

else

delta=previousADC-v;

if (delta>10)

{

sleepCounter=0;

}

previousADC=v;

// VOLTAGE CALCULATION

b=v*10.676-5311;

b=b/10;

if (b<0.0)

{

StartMaskCom=0x08;

b=-b;

}

else

StartMaskCom=0;

v=(TWord)b;

if (v<2)		//    BEEP if (v<2);

{

ICR1=25;

TCCR1B=0x19;

}

else			// stop BEEP

TCCR1B=0;

// SET VOLTAGE VALUE TO LCD BUFFER

lcdBuffer[0]=v/100;

v-=lcdBuffer[0]*100;

lcdBuffer[1]=v/10;

v-=lcdBuffer[1]*10;

lcdBuffer[2]=v;

}

void main(void)

{

TWord adc;

DeviceInit();

LowBeep();

HighBeep();

for (;;)

{

//check sleep

if (sleepCounter>150*60)    //60sec

{

HighBeep();

LowBeep();

ADMUX=0;

ADCSR=0;

DDRB=0xF2;

PORTB=0xF2;

PORTC=0xFB;

PORTD=0xFF;

//go to power-down

MCUCR|=(1<<SE)|(1<<SM1);

__sleep();

}

if (adcCounter<150/20)

continue;

adcCounter=0;

ADMUX=0xc2;

//enable ADC and reference

ADCSR=(1<<ADEN)|(1<<ADSC)|(1<<ADPS0);

//wait for the dummy

while (!(ADCSR&(1<<ADIF)));

//clear flag

ADCSR|=(1<<ADIF);

//run the ADC

ADCSR|=(1<<ADSC);

//wait for the result

while (!(ADCSR&(1<<ADIF)));

//clear flag

ADCSR|=(1<<ADIF);

//read the value and convert

adc=ADC;

ADMUX=0;

ADCSR=0;

if (adc>999)

adc=999;

ConvertResult(adc);

LCD_UpdateBuffer();

}

}

void SetSegments(TWord data)

{

if (data&0x0001)

macBitOn(PORTD, PIN1);

else

macBitOff(PORTD, PIN1);

if (data&0x0002)

macBitOn(PORTD, PIN2);

else

macBitOff(PORTD, PIN2);

if (data&0x0004)

macBitOn(PORTD, PIN3);

else

macBitOff(PORTD, PIN3);

if (data&0x0008)

macBitOn(PORTD, PIN4);

else

macBitOff(PORTD, PIN4);

if (data&0x0010)

macBitOn(PORTC, PIN5);

else

macBitOff(PORTC, PIN5);

if (data&0x0020)

macBitOn(PORTD, PIN6);

else

macBitOff(PORTD, PIN6);

if (data&0x0040)

macBitOn(PORTC, PIN7);

else

macBitOff(PORTC, PIN7);

if (data&0x0080)

macBitOn(PORTC, PIN8);

else

macBitOff(PORTC, PIN8);

if (data&0x0100)

macBitOn(PORTD, PIN9);

else

macBitOff(PORTD, PIN9);

if (data&0x0200)

macBitOn(PORTD, PIN10);

else

macBitOff(PORTD, PIN10);

}

//================================================

//  timebase interrupt and refresh LCD

//================================================

#pragma vector=TIMER0_OVF_vect

__interrupt void IntHandlerT0(void)

{

TCNT0=256-31;	//  interrupt with freq=(37000/8/31)=149Hz

sleepCounter++;	// increment sleep counter

adcCounter++;	// increment ADC counter

LCD_com_counter++;	// increment phase counter

if (LCD_com_counter>=6)

{

LCD_com_counter=0;

}

//SET COM1 COM2 & COM3

if (LCD_com_counter==0)		//PHASE0

{

macPinOff(DDRB,COM2);

macPinOff(DDRB,COM3);

macPinOn(PORTB,COM2);

macPinOn(PORTB,COM3);

macPinOn(DDRB,COM1);

macPinOff(PORTB,COM1);

SetSegments(DataCom1);

return;

}

if (LCD_com_counter==1)		//PHASE1

{

macPinOff(DDRB,COM1);

macPinOff(DDRB,COM3);

macPinOn(PORTB,COM1);

macPinOn(PORTB,COM3);

macPinOn(DDRB,COM2);

macPinOff(PORTB,COM2);

SetSegments(DataCom2);

return;

}

if (LCD_com_counter==2)		//PHASE2

{

macPinOff(DDRB,COM1);

macPinOff(DDRB,COM2);

macPinOn(PORTB,COM1);

macPinOn(PORTB,COM2);

macPinOn(DDRB,COM3);

macPinOff(PORTB,COM3);

SetSegments(DataCom3);

return;

}

if (LCD_com_counter==3)		//PHASE3

{

macPinOff(DDRB,COM2);

macPinOff(DDRB,COM3);

macPinOn(PORTB,COM2);

macPinOn(PORTB,COM3);

macPinOn(DDRB,COM1);

macPinOn(PORTB,COM1);

SetSegments(~DataCom1);

return;

}

if (LCD_com_counter==4)		//PHASE4

{

macPinOff(DDRB,COM1);

macPinOff(DDRB,COM3);

macPinOn(PORTB,COM1);

macPinOn(PORTB,COM3);

macPinOn(DDRB,COM2);

macPinOn(PORTB,COM2);

SetSegments(~DataCom2);

return;

}

if (LCD_com_counter==5)		//PHASE5

{

macPinOff(DDRB,COM1);

macPinOff(DDRB,COM2);

macPinOn(PORTB,COM1);

macPinOn(PORTB,COM2);

macPinOn(DDRB,COM3);

macPinOn(PORTB,COM3);

SetSegments(~DataCom3);

}

}

CPORTLCD.H

#ifndef _cportLCD_H

#define _cportLCD_H

#define macPinOn(PORT,PIN)\

PORT|=(0x01<<PIN)

#define macPinOff(PORT,PIN)\

PORT&=~(0x01<<PIN)

//===================================================================

// PORT B

//===================================================================

#define COM1			0x00

#define COM2			0x02

#define COM3			0x03

#define BEEPER		0x01

//===================================================================

// PORT C

//===================================================================

#define INPUT_ADC	0x02

#define PIN8			0x03

#define PIN7			0x04

#define PIN5			0x05

//===================================================================

// PORT D

//===================================================================

#define PIN1			0x00

#define PIN2			0x01

#define PIN3			0x02

#define PIN4			0x03

#define PIN6			0x05

#define PIN9			0x06

#define PIN10			0x07

#endif

DEFINES.H

#ifndef _DEFINES_H

#define _DEFINES_H

#ifndef __BORLANDC__

#define FALSE	0

#define TRUE	!FALSE

typedef unsigned char bool;

#define false	0

#define true	!false

#endif

#define	INPUT	1

#define OUTPUT	0

#define	HIGH	1

#define	LOW	0

#define	OUTPUTS	0x00

#define	INPUTS	0xFF

#define ON	TRUE

#define OFF	FALSE

#define HiByte(x) (TByte)(x>>8)

#define LoByte(x) (TByte)(x & 0xFF)

//use ~0 instead of 0xFF

#define BitsOn(var,mask) var|=mask

#define BitsOff(var,mask) var&=~0^mask

#define macBitOn(REG,BIT) (REG|=(0x01<<BIT))

#define macBitOff(REG,BIT) (REG&=(~(0x01<<BIT)))

#define macTestBit(REG,BIT) (REG&(1<<BIT))

/***********************************************

***********************************************/

#ifdef __BORLANDC__

#define TRUE    true

#define FALSE   false

typedef unsigned __int8 TByte;

typedef TByte *TBytePtr;

typedef unsigned __int16 TWord;

typedef TWord *TWordPtr;

typedef unsigned __int32 TDWord;

typedef TDWord *TDWordPtr;

typedef signed __int32 TLongInt;

typedef TLongInt *TLongIntPtr;

#else

typedef unsigned char TByte;

typedef TByte *TBytePtr;

typedef unsigned int TWord;

typedef TWord *TWordPtr;

typedef unsigned long int TDWord;

typedef TDWord *TDWordPtr;

typedef signed long int TLongInt;

typedef TLongInt *TLongIntPtr;

#endif

#ifdef __IAR_SYSTEMS_ICC__

typedef double TReal;

typedef TReal *TRealPtr;

#elif HI_TECH_C

typedef double TReal;

typedef TReal *TRealPtr;

#else

typedef float TReal;

typedef TReal *TRealPtr;

#endif

#define TBCD	TByte

#ifndef __IAR_SYSTEMS_ICC__

#define flash

#define C_task

#endif

#endif