Name 			 Value

eProDas_Index_PORTA  		0

eProDas_Index_PORTB  		1

eProDas_Index_PORTC  		2

eProDas_Index_PORTD  		3

eProDas_Index_PORTE  		4

int eProDas_OpenHandle();    void eProDas_CloseHandle();

int eProDas_Initialize(unsigned int DeviceIdx);

int eProDas_ConfigurePort(unsigned int DeviceIdx, unsigned int Port,

unsigned char NewTris, unsigned int Action);

int eProDas_ConfigurePorts(unsigned int DeviceIdx, unsigned char *NewTrises,

unsigned int *Actions);

int eProDas_ConfigurePinAsOutput(unsigned int DeviceIdx, unsigned int Port,

unsigned int Pin, unsigned int InitialState);

int eProDas_ConfigurePinAsInput(unsigned int DeviceIdx, unsigned int Port,

unsigned int Pin);

nt eProDas_ConfigurePortUnsafe(unsigned int DeviceIdx, unsigned int Port,

unsigned char NewTris);

int eProDas_ReadPort(unsigned int DeviceIdx, unsigned int Port,

unsigned char &Value);

int eProDas_ReadPorts(unsigned int DeviceIdx, unsigned char *Values);

int eProDas_ReadSelectedPorts(unsigned int DeviceIdx, unsigned int PortFlags,

unsigned char *Values);

int eProDas_WritePort(unsigned int DeviceIdx, unsigned int Port,  unsigned char NewValue);

int eProDas_WritePorts(unsigned int DeviceIdx, unsigned char *NewValues);

int eProDas_WriteSelectedPorts(unsigned int DeviceIdx, unsigned int PortFlags,

unsigned char *NewValues);

int eProDas_ReadWriteSelectedPorts(unsigned int DeviceIdx, unsigned int ReadFlags,

unsigned int WriteFlags, unsigned char *ReadValues, unsigned char *WriteValues);

int eProDas_SetPin(unsigned int DeviceIdx, unsigned int Port, unsigned int Pin);

int eProDas_ClearPin(unsigned int DeviceIdx, unsigned int Port, unsigned int Pin);

int eProDas_TogglePin(unsigned int DeviceIdx, unsigned int Port, unsigned int Pin);

int eProDas_TogglePinTwice(unsigned int DeviceIdx, unsigned int Port,

unsigned int Pin);

int eProDas_ConfigureInternalAD(unsigned int DeviceIdx,

unsigned int NumberOfADPorts, unsigned int AcquisitionTimeCode,

unsigned int VrefPlus, unsigned int VrefMinus);

AD channel  Physical pin

AN0	RA0

AN1 	RA1

AN2 	RA2

AN3 	RA3

AN4 	RA5

AN5 	RE0

AN6 	RE1

AN7 	RE2

AN8 	RB2

AN9 	RB3

AN10 	RB1

AN11 	RB4

AN12	 RB0

AcquisitionTimeCode  AD clock periods  Acquisition time [us]

0 		0 		 0

1 		2  		2,7

2  		4  		5,3

3  		6  		8,0

4  		8  		11

5 		12  		16

6 		16  		21

7  		20  		27

int eProDas_ConfigureInternalAD_Advanced(unsigned int DeviceIdx,

unsigned int NumberOfADPorts, unsigned int AcquisitionTimeCode,

unsigned int VrefPlus, unsigned int VrefMinus, unsigned int Justification,

unsigned int FullAccuracy);

int eProDas_AcquireInternalADChannel(unsigned int DeviceIdx, unsigned int ADPort);

int eProDas_ReadInternalADChannel(unsigned int DeviceIdx, unsigned int &Result);

int eProDas_AcquireAndReadInternalADChannel(unsigned int DeviceIdx,

unsigned int ADPort, unsigned int &Result);

int eProDas_ReadAndAcquireInternalADChannel(unsigned int DeviceIdx,

unsigned int &Result, unsigned int ADPortToAcquireAfter);

int eProDas_ReadSelectedInternalADChannels(unsigned int DeviceIdx,

unsigned int ChannelFlags, unsigned int *Results);

int eProDas_ConfigureInternalReference(unsigned int DeviceIdx,

unsigned int Enabled, unsigned int Value, unsigned int Range,

unsigned int SourceOnPin, unsigned int OutputOnPin);

int eProDas_ConfigureInternalPullupResistors(unsigned int DeviceIdx,

unsigned int Enable);